A CELLULAR TUTORIAL

Copyright (C) 1994 by Network Wizards

This document may be printed for reading by the user of this kit,

or copied to other media for backup purposes.  It may not be

reproduced or retransmitted in any other form for any other purpose.

February 10, 1994

OVERVIEW

The cellular telephone system is made up of land stations (base stations)

and mobile stations (cellular telephones).  Each land station provides

service coverage to a small geographic area and is called a cell.  Clusters

of land stations are interconnected to provide continuous coverage as the

mobile moves from cell to cell.  Land stations are connected to a central

computer system that controls the cellular network in a particular area.

The cellular network connects to the public telephone network to provide

incoming and outgoing call access.

SERVICE PROVIDERS

A group of land stations in a particular area are controlled by a single

cellular service provider.  Each geographic area is allowed to have two

service providers, licensed by the FCC, and known as the Wireline or

Non-Wireline carriers.  They are also referred to by which system or set of

frequencies they operate on, named the A (non-wireline) and B (wireline)

systems.

Each service provider's area is assigned a unique 15-bit system id (SID).

The SID is odd for A systems and even for B systems.  The two highest order

bits of the SID are defined as a country code as follows:

Bit 14    Bit 13      Country

0	    0		United States

0	    1		Others

1	    0		Canada

1	    1		Mexico

CELLULAR CHANNELS

Each cellular system uses an assigned frequency range.  Mobile units

communicate with base stations using a pair of frequencies, know as the

channel number.  Mobile units transmit on a frequency that is 45mhz lower

than the frequency base stations transmit on.  Mobiles transmit in the

824-849mhz range, and base stations transmit in the 869-894mhz range.  So,

for example, channel 1 is assigned frequency pair 825.030/870.030mhz.  The

mobile transmits on 825.030mhz, and the base station receives on this

freqency.  The base transmits on 870.030mhz and the mobile receives on it.

Each cellular system is assigned 416 channels, for a total of 832 cellular

channels.  Within each system, 21 channels are used as control channels,

leaving 395 channels for actual telephone calls.  Each cell site will

normally have at least one control channel, and some number of voice

channels.  Neighboring cell sites use different channels to avoid

interference.  Cell sites are connected to a central control center for the

cellular system, known as the Mobile Telephone Switching Office (MTSO),

usually over leased land lines or microwave circuits.

The A system is assigned channels 1-333, 667-716, and 991-1023.  The B

system is assigned channels 334-666 and 717-799.  System A control channels

are 313-333, and system B control channels are 334-354.  Its a mess because

the cellular system originally covered only 666 channels, and 166 were

added on later, both before and after the original frequency allocation

block.  Channel numbers can be converted to frequency with the following:

Transmitter	Channel		Frequency

Mobile	1-799		825.000+0.03*C

990-1023	825.000+0.03*(C-1023)

Land		1-799		870.000+0.03*C

990-1023	870.000+0.03*(C-1023)

CELLULAR CONTROL

The MTSO controls operation of the cellular system, sends orders to mobile

units and receives requests from them.  Control channels are used for

sending and receiving orders.  The control channel is usually busy

transmitting pages (or incoming call requests) to mobile units.  The land

station also transmits overhead information, such as the system id and

global parameter and action requests.

When a mobile unit is turned on, it scans the assigned control channels of

the appropriate system (A/B) and locks onto the strongest channel.  It

then waits until it sees itself being paged, or until the user requests to

make an outgoing call.

If the phone sees itself being paged, it sends back a message indicating

it can receive a call.  The land station then assigns a channel and sends

back a message telling the phone to tune to that channel.  The land

station then tells the phone to starts ringing.  When the user presses

SEND to answer the call, it sends a message back to the control channel

saying it will accept the call, and voice communication then begins.

If the user wants to make an outgoing call, the number to dial is sent on

the control channel when the user presses SEND.  The MTSO then assigns an

empty voice channel in the current cell for the call, and sends back a

message on the control channel telling the mobile what channel number was

assigned to it.  The mobile then tunes to that channel and the call begins.

If the mobile begins moving out of the current cell, the MTSO will notice

its signal strength getting weaker.  It will check neighboring cells to

see if the signal is getting stronger in another cell.  If so, the MTSO

will command the mobile to hand-off to a channel in the new cell.  It does

this by sending a new channel number over the voice channel.  The mobile

acknowledges the command and both the mobile and MTSO switch over to the

new channel.  This process continues until the call is completed.

MOBILE IDENTIFICATION

Each mobile unit is uniquely identified by its Electronic Serial Number

(ESN).  In addition, each mobile may have any number of telephone numbers

assigned to it, known as Number Assignment Modules (NAMs).  Each NAM

contains a telephone number, system id, group id mark, access overload

class, and initial paging channel information.  The telephone number is a

standard 10-digit number made up of area code, prefix, and number.  The

system id identifies which cellular system provider issued the telephone

number.  When telephone calls are initially set up, the ESN and NAM

are sent from the mobile unit to the base station for verification.

The ESN is a 32-bit number:  the first 8 bits identify the telephone

manufacturer, and the remaning bytes indicate a serial number.  The NAM

telephone number is represented as a Mobile Identification Number (MIN) in

34 bits, divided into MIN1 and MIN2 pieces.  MIN2 is 10-bits and represents

the encoded area code, and MIN1 is 24-bits and represents the encoded

prefix and number.

ESN First Byte (hex)		     Manufacturer

81				OKI Telecom

82				Motorola

85				Fujitsu

86				Mitsubishi

87				NEC

88				Panasonic

8A				Audiovox-Audiotel

8D				Goldstar

8E				Novatel

8F				Ericsson

94				Blaupunkt

96				Alpine

9A				Sony Corp.

9C				Mobira

9D				Ericsson GE

9F				Qualcomm, Inc.

A2				Technophone

A5				Nokia/Tandy/Mobira

AC				Uniden

MOBILE STATIONS

Mobile stations are classified into three categories depending on their

maximum effective radiated power (ERP).  They are:

Class I		6 dBW (4.0 Watts)

Class II	2 dBW (1.6 Watts)

Class III      -2 dBW (0.6 Watts)

Mobile stations can be commanded by a base station to reduce their power

output from the maximum in seven steps, depending on the Class of the

mobile unit.  The power levels are:

Mobile Attenuation Code		ERP (dBW) for Class

(MAC)			  I	 II	III

0				  6	  2	  2

1				  2	  2	 -2

2				 -2 	 -2	 -2

3				 -6	 -6	 -6

4				-10	-10	-10

5				-14	-14	-14

6				-18	-18	-18

7				-22	-22	-22

A field known as the Station Class Mark (SCM) is sent from the mobile

to the base to identify the type of mobile unit.  This four bit field

is divided into three parts:

Power Class	    Transmission	      Bandwidth

Class I	   xx00	      Continuous     x0xx	20 MHz    0xxxx

Class II    xx01	      Discontinuous  x1xx	25 MHz    1xxxx

Class III   xx10

Reserved    xx11

The bandwidth bit is normally set.  Older cellular phones that were built

before the freqency range was expanded from 666 to 832 channels have this

bit cleared.

Each mobile station is programmed with a 4-bit field called the Access

Overload Class.  If the cellular system gets overloaded, it can command

particular mobile stations to not attempt to access the system.  It does

this by transmitting a 16-bit bit-mapped field indicating which Classes

should not attempt access.  Normally this field is set to the last digit

of your telephone number (0-9).  Class 10 is recommended for test mobile

units, and class 11 for emergency units.  Classes 12-15 are reserved.

SUPERVISORY AUDIO TONE (SAT)

The supervisory audio tone is one of three frequencies listed below.  The

SAT is added to the voice transmission by a land station.  A mobile station

detects, filters, and modulates the transmitted voice channel carrier with

this tone.  The tone is used to detect interference between neighboring

channels.  The SAT is not transmitted when sending wideband data.

SAT Code       Frequency

0		5970hz

1		6000hz

2		6030hz

SIGNALLING TONE

The signalling tone is a 10khz tone.  It is sent by the mobile station

on a voice channel to confirm orders, signal flash requests, and signal

release requests.

WIDEBAND DATA MESSAGES

Land and mobile stations can communicate control information over both

control and voice channels.  If the land station is transmitting, it is

called the Forward channel, and if the mobile is transmitting, it is called

the Reverse channel.  There are four possibilities, known as the Forward

Control Channel (FCC), Forward Voice Channel (FVC), Reverse Control Channel

(RCC), and Reverse Voice Channel.  Data is sent at a 10khz rate, repeated

multiple times, and encoded with error checking and correcting information.

ORDERS

A land station can send the following orders to a mobile station.  These

are sent in a wideband data stream on either the control or voice channels.

Alert		Informs the user that a call is being received.

Audit		Determines if a mobile station is currently active.

Change Power	Orders the mobile to change RF output power level.

Intercept	Informs the user of error made in placing call.

Maintenance	Used for testing; similar to alert.

Page		Informs mobile of an incoming call request.

Release		Disconnects a call being established or already established.

Registration	Requests mobile to register its identity with the system.

Reorder		Informs the user that all facilities are in use.

Send Called-Address	Orders the mobile to send dialed-digit information.

Stop Alert	Informs mobile to stop alerting the user.

THE CELLULAR STANDARD

The cellular telephone standard is documented as standard EIA/TIA-553,

September 1989, "Mobile Station - Land Station Compatibility

Specification".  It documents the complete protocol and messaging formats

used by the cellular system.  It is published by:  Electronic Industries

Association;  2001 Pennsylvania Ave, N.W.;  Washington, DC 20006.  It can

also be ordered from many standards distributors.  One such place located

in the San Francisco Bay Area is Document Center (Belmont, CA) at (415)

591-7600.

OTHER BOOKS

The Cellular Travel Guide contains information about cellular phone

service in the US and Canada.  It contains coverage maps, information

on roaming, airtime charges, access numbers, and more.  Its currently

available for $19.90 from Communications Publishing Service at

(800) 366-6731.  They also sell other cellular related books.

INSIDE THE OKI-900

The "Technical Instruction Manual for OKI Phones 900 Handheld Cellular

Telephone (Model UM9022)", part# 01-20001, can be ordered from OKI Telecom,

at (800) 554-3112.  It contains maintenance and tuning procedures,

schematics, and detailed operating instructions.

The OKI-900 contains two printed-circuit boards: one is a radio board, and

the other the digital control board.  The 900 uses two microcontrollers,

one to control the cellular telephone operation, and another to control the

LCD display and keypad.  The two processors communicate with each other

over a three-wire serial link.  The control processor can also communicate

with external devices through the jack on the bottom of the phone.

The display/keypad controller is an 80C154, with its control program in

internal ROM.  The main controller is an OKI variant of the 80C51.  This

8051 has 256 bytes of internal RAM, an 8 channel A/D converter, and 6 8-bit

I/O ports (2 for address/data bus, 1 for A/D, 3 for general purpose I/O).

The 8051 uses an external ROM (64K 27C512) for program storage.  It uses an

external EEPROM (8K 28C64) for storing memories and configuration

parameters.

The program ROM is mapped into the 8051's program address space.  The

EEPROM is mapped into the 8051's memory address space, along with some

other devices.  These include an 81C55 port expander, which gives the

processor 256 more bytes of RAM, and 3 more 8-bit I/O ports.  A DTMF

decoder chip is connected to one of these ports, and the rest control or

monitor various parts of them phone.

INSIDE THE OKI-1150

There is also a technical manual for the OKI-1150 available from OKI.

It contains the same type of information as the OKI-900 manual.  The

OKI-1150 is built on a single printed-circuit board.  It uses a

proprietary OKI processor that is not compatible with any other

processors (although kind of similar to an 8051).  It uses an 80C154

processor for controlling the display and keypad.

The main processor (an MSM65x227) contains many I/O lines, a built-in

A/D converter, and 4K bytes of internal EEPROM which are used for

storing memories and configuration data.  An external ROM holds the

program code.